NEW PORT FACILITY FOR THE FALKLAND ISLANDS: FREQUENTLY ASKED QUESTIONS FOLLOWING THE SUBMISSION OF PLANNING APPLICATION FOR THE NEW PORT.  MARCH 2022

| COMMENT / TOPIC AREA | RESPONSE |
| --- | --- |
| Questions on Facilities being provided |  |
| 1 Is the port future proof for economic growth and will it be big enough for rapid cargo transfer and expected vessel number increases ? | The project calculated the throughput capacity of the New Port Facility( NPF) by undertaking extensive engagement with all industry sectors that use the port, to build a market forecast of future demand. BAM also undertook an independent assessment as part of this work called the Demand Study. The current FIPASS shipping schedule was assessed and analysed to understand patterns of arrival and departure. FIPASS at present has two main operational berths (central and eastern berths), each around 90m in length, with an additional eastern auxiliary berth directly at the end of the access link bridge which is known as the eastern Ro-Ro berth. A third berth was available on the western end of the facility but has since been decommissioned in November 2018 due to its failing structural condition. In comparison, the NPF will provide 300m of continuous quay, a separate leeward berth on the southern face, and also an eastern berth. This will provide for a minimum of three operational berths compared to the two currently available at FIPASS. There can be up to four or five berths in certain managed scenarios available for some of the smaller vessels if scheduling / demand requires this in busy transhipment windows, and also future potential to expand if required. Using current shipping schedules and patterns and mapping this over a minimum new three-berth set up, the berth occupancy rates assessed showed that there will be spare berth occupancy available to allow for future cargo growth. This was based on a maximum berth occupancy rate of 60% per annum. Information on indicative berthing layouts is given below in Answer 2. This demonstrates that with this layout it is possible to accommodate the future high cargo growth scenario projected in the Demand Study for all cargo types. However, it should be noted that for all cargo types to follow the high case cargo growth trajectories from the Demand Study is quite optimistic. Nonetheless, the most ‘optimistic’ vessel volume has been used to deduce the design. The layout will provide sufficient space to operate safely and to accommodate the present cargo throughput as well as having the ability to serve larger future growth. A robust vessel schedule management system will be in place when the new port facility opens, which will manage busy periods effectively. It will be based on average call durations and parcel sizes. Variable pricing scales could also be used to encourage use of the Southern Berth where possible for inspection / layover etc to avoid blocking the main north face. Given the need to balance capital expenditure and functionality for the NPF, the main consideration is to design a facility which is efficient and meets projected demand, without compromising ability to grow capacity in the future. Should additional growth emerge in the very long term, there are options to expand the berth capacity by installing a western mooring dolphin. This would allow for more berthing space on the remaining jetty whilst a large vessel moors up. In addition, the southern berth and the approach channel could be deepened further to allow for larger vessels to berth alongside. There also remains the potential to extend the main quay if growth exceeded all the current projections to 2050 and beyond. Growth of this magnitude would significantly increase GDP. The NPF will offer a substantially improved port facility to the current FIPASS. The infrastructure and quay in the future will allow the utilisation of more efficient cargo handling equipment, which in turn will allow the assumptions used for the port capacity calculations to be improved upon. Ship turn-around times should improve, which will in turn increase the theoretical berth capacity and business efficiency for port users. Indicative Berthing scenarios are attached as drawings in Appendix 1 for further information – see illustrations below. |
| 2 How are multiple vessels allowed for in the design? | The NPF design needs to cater for a wide range of cargo and services in the future. The need to accommodate all services using FIPASS currently was a core part of the Project Requirements. The final length of the main north berthing face is considered to be the most balanced in terms of operational functionality and affordability, at 300m. Many permutations of simultaneous berthing configurations over the length of a 300m continuous north berth face, including multiple fish transhipment scenarios with varying plant, and various scenarios with cruise ships, fishing vessels, MV Scout etc have been modelled to ensure the port can accommodate these, noting that some of these permutations are not frequent patterns of berthing. Some indicative scenarios are appended to this document as Appendix A. The Sir David Attenborough vessel is not part of the design fleet of vessels considered for the New Port facility as its draught requirements cannot be accommodated. The issue is the limited available draught between The Narrows and the new port. The Sir David Attenborough has significant under keel clearance requirements due to its scientific instrumentation on the underside of the vessel. The limitations are indicated on the latest UKHO chart below with the new facility transposed. |
| 3 What are the Facilities for yachts and launches? | It is envisaged that large yachts that stay for more than a day or so can moor at the Southern berth. The Concordia Bay will use this berth as well if the northern berth is busy but as the Concordia Bay visits Stanley once every 6 weeks, this is not seen as an issue that cannot be managed. The 40m eastern berth is also available for some yachts / tugs / large launches etc There are 90m of pontoons provided to the east of the access causeway. Of this 90m, one 20m pontoon is capable of carrying the loads from a telehandler so that resupply launches can be loaded for vessels at anchor in the harbour. At this new heavy duty pontoon, will be a diesel refuelling dispenser and tank for this fuel ( managed by SSL) and additional bunkering services available for all users (water and power, firefighting equipment and lighting ). A further 70m of reused lightweight pontoons will cater for the existing launches and small boats that use this facility already. Options for a marina at a later stage on the south side have been considered and could be accommodated if a suitable Business Case is progressed – The current scheme has passive design capacity only in order to focus on delivering the essential project requirements in the first instance. This is therefore not within the scheme submitted for Planning determination , but the design does not preclude this being provided at a later stage. |
| 4 Why is a ro-ro not included on the Quay? | The decision to exclude from the scope to optimise a flexible south berth face, was made to design for growth of all market sectors. At concept design stage the provision of the ro-ro ramp was carefully considered for design development. Assessing costs against benefits, it was assessed as not representing best value to provide this functionality, as it would not be heavily utilitised. There is one current ship (Concordia Bay) that uses a RORO and no identified future vessel that would only be able to berth using a RORO facility. The omission of a ro-ro ramp greatly increases flexibility of the port and allows for use of this berth by many other vessels; providing potential layover and also increasing berth availability on the north face. The southern and northern berths are available for use by the Concordia Bay. It does not need a ro-ro to berth at NPF. The intent is for the existing FIPASS Safe System of Work to continue, as used successfully until now. Loading by crane works for the Concordia Bay, as it has a longer dwell time at FIPASS and thus the NPF than at other locations. A review of any possible timetable amendments was also undertaken and indicated the infrequent berthing pattern will continue with the current (and any future similar) vessel. It should also be noted that using New Haven for RORO cargo to island destinations results in reduced steaming time (and thus emissions) for the vessel and is far more efficient overall in almost every scenario. Additionally, as a landing craft, the Concordia Bay is able to load over very simple beach ramps and for the rare occasions where a RO-RO operation is useful in Stanley, is currently able to utilise the reclaimed land in front of the museum The scope without the RO-RO was validated by Executive Council in September 2021, and the design and planning application has been developed and issued for approval on this basis. |
| SCUTTLING |  |
| 5 Why is Scuttling not in the application and dismantlement on the shore is proposed | The project has considered multiple alternative options for the decommissioning / disposal of FIPASS, one of which is scuttling at sea, and another of which is interest from the private sector internationally in removing the barges for commercial resale. The option which is included in the planning application is dismantling on the foreshore, as this is the option that the project team are confident can be implemented at this stage in the project, on consideration and balance of Health & Safety risks, costs, programme impact and technical risks. The solution allows environmentally for the recycling of the materials and the safe disposal of the hazardous materials and also manages the H & S risks identified in the EIS, i.e. asbestos and the contaminated sludge. FIG and BAM are in parallel undertaking an option appraisal for scuttling. Although scuttling of barges at sea is not the proposed solution at present (and has not, therefore, been documented in the planning application and the EIS), the option has not been ruled out. If this appraisal confirms it is viable, both from an environmental and cost perspective, then a separate planning application and EIS would be issued to assess this option fully. Technical risks being assessed at this point include control of invasive species, navigation, storage in harbour risks, as well as the suitability and ecology impact of the possible disposal site. In addition, the barges might not be able to be cost effectively towed out through the Narrows, due to the condition of the barges themselves, without significant structural amendment. The costs of making them sea worthy for towing to a suitable scuttling site may or may not be considered feasible by FIG when all current assessments are concluded and considered by FIG in a few months’ time. |
| Queries on Road, Vehicle usage & Traffic |  |
| 6 What are the impacts to Stanley Growers and mitigation measures proposed? | Discussions are ongoing between FIG and Stanley Growers (SGL), with FIG’s intention being to ensure that any loss of land (either temporarily during the construction stage or long term when the new port becomes operational) could be offset by the offer of provision of suitable alternative land at a suitable nearby location, and to also assist with the re-siting or re-provision of the affected polytunnels. In addition, it is proposed to offer to enhance agricultural areas on the remaining SGL land through the relocation of the existing topsoil and peat generated from excavations to construct the access road and preparation of the dismantlement site. The plan is not to create new topsoil from first principles but to carefully re-use what has been improved already. FIG and BAM are working with SGL to understand what can be done out of growing seasons to mitigate / address any potential negative effect commercially to SGL. On this basis, while acknowledging that there will inevitably be disruption and impact to SGL, with FIG’s intention to offset this disruption by the provision of suitable alternative land for agricultural food production at a nearby location and support in the re-provision of polytunnels as required, the environmental impact on the wider community / natural world in the EIS is predicted to be low ( ie negligible as defined at scoping opinion stage). |
| 7 Why was the proposed Access Road route chosen and what are the potential impacts to the irrigation pond ? | A full review of all options, including a north road alignment was undertaken in Stage 1A. A balanced decision was made on what was best for project based on technical complexity, cost, impact on national infrastructure, and this included an assessment of the effect on SGL and other stakeholders. The lower route close to the coast was best for minimising the disruption to SGL but was very disruptive to existing services, as would have involved lowering strategic infrastructure, including the foul pumping main for Stanley, the HV supply to other FIPASS road businesses and the local water main. These lowering works would have been over hundreds of metres and would have been very expensive. The road would have also required retaining structures, close to the beach, affecting most of the length of foreshore and significant additional quarry material due to the topography. When all of these factors were considered and balanced, the proposed route was the most advantageous. We note the previous work undertaken by Stanley Growers to the irrigation pond to increase its size and to enhance the habitat around it. The pond and its surrounding habitat is maintained in the design and protected in the construction stage. As detailed in Section A17.2.3, the pond will not be directly altered by the proposed scheme. The proposed works will however temporarily interact with the flow of water in and out of the irrigation pond during construction of the access road (as the proposed access road will travel over the drainage channel that feeds the irrigation pond). The new culverts will be installed ahead of the old one being stopped up to ensure that water can continue to flow into the pond and be abstracted for irrigation purposes. Existing IBC storage will be removed and re-sited to reduce pollution risks. Section A17.4.2 states that the pond will not be directly altered during the operational phase, although it could potentially be affected by surface water drainage from the proposed access road. The design of the access road has taken into account the potential for risk of surface water flooding to the road and the effect that the road may have on existing surface and groundwater flow pathways. Suitable drainage measures have been incorporated into the design of the access road, including permeable surfacing and a highway drainage network feeding into a swale system. As a result, no impact to the pond is predicted during operation. |
| 8 Why is traffic shown on the MPA Road and why is Mare Harbour used in the EIS and not FIPASS? | It obviously makes best sense for the project and FIG to aim to use Stanley Harbour for unloading of the maximum amount possible of cargo. In the current procurement that BAM are undertaking for sub-contracts, the steer to potential suppliers has been that FIG and BAM want as much cargo delivered directly to Stanley as possible to cut down on logistics from Mare Harbour, both from a cost perspective and also taking into account the concerns about heavy traffic on the roads. It is not feasible to import all construction plant / equipment to site via FIPASS however, given current weight restrictions on FIPASS and the size of some of the components needed. Where that may be the case, the safe option would be to import those components to Mare Harbour and transport to site as abnormal loads where necessary. As the EIS needs to reflect ‘worst possible case’ from an environmental impact perspective especially for emissions and noise and is ahead of completion of procurement activities, all imports are currently shown from Mare Harbour. As detailed in Section A4.2.3 of the EIS, it is recognised that traffic movements from Mare Harbour would have a very short term impact on existing traffic along the route until all plant / machinery is transported (with consequent impacts to air quality and generation of noise). Once procurement has concluded, BAM will put forward their proposal to FIG around logistics and shipping tender / supplier in the coming weeks, with the aim to find the optimum solution for delivering all required cargo to the work site and for reducing emissions and traffic further. Note that Traffic movements generated by this activity would not affect the ‘peak traffic flows’ for construction on which the assessment has been based because the peak traffic flow occurs at a later stage in the construction programme. The effect would be on emissions and length of each journey. |
| 9 HGV turning and car parking | There is a turnaround area just before the gatehouse which has an acceptable swept path for an HGV and is as per requirements agreed with the current port operator for safe systems of work and security protocols. A port should not be sized for significant dedicated carparking as costs will rise significantly and this will compromise value for money. The port will have a flexible parking plan managed by the port operator, based on berthing activity and H & S management and will consider day and night activities separately. The intention of the port layout is to keep to a minimum the number of ‘dedicated’ private vehicles bays on the port to ensure a flexible and adaptable berthing arrangement. A small amount of dedicated car parking provision will be made available on the new port for the number of port management staff working permanently on the port. The size of the port is set to meet the demands of port operations. Working practices may need to change from what takes place on FIPASS currently. Additional carparking permissions for berthing activities for specific vessels are dependent on transhipment volumes on the day, other vessels in port and are at the discretion of the port operator. This will continue to be managed by the port operator in the future, depending on berthing schedules and operations on any one day. With regard to the temporary accommodation block car parking has been set based on the use of minibuses for transportation to site, as commonly used by BAM world-wide for our workforce and also by other contractors on the Falklands to minimise emissions, traffic and costs for transportation. Safe pedestrian routes to and from the block will be agreed with PWD as part of the road works design and will be agreed as part of clearance of conditions. |
| Effects & Queries from Other Stakeholders in proximity to the new port |  |
| 10 What are the Impacts on other businesses including Seafarer’s Mission ? | FIG will work with the Seafarer’s Mission to find suitable alternative locations for the four containers of clothes for emergency response currently stored in the proposed offices / plant workshop area affected in the construction phase only. Multiple alternative options are available to store these containers in the vicinity. Relocation of the Seafarer’s Mission is not within the application as this is not being proposed as part of the scheme. Demolition and relocation of the Seafarer’s Mission to construct the road was not considered as part of the conceptual options for the new access road, as it is not directly affected by the construction works. Liaison with the Seafarer’s Mission will continue through the next few months to mitigate disruption to its services during the construction stage. This applies to all affected landowners / tenants. FIG and BAM will work with landowners / tenants in order to minimise disruption to ongoing practices as far as practicable. It is noted that Seafarers will have a longer journey to walk to the Seafarer’s Mission during the operational stage compared to the current situation. However, during operation, Seafarers within the mission will be subject to less disturbance compared to the present day situation, as vehicles will not be travelling immediately adjacent to the Mission to transport goods on and off FIPASS. During the baseline noise survey, the FIPASS causeway was noted to be a major contributor to the ambient noise level. As detailed in Section A15.4.2, the views out to sea from the Seafarer’s Mission will also change through the removal of the existing causeway and the degrading FIPASS structures; this is considered to be a beneficial impact during the operational phase due to the degrading condition of FIPASS. From an operational noise perspective, an assessment of the difference between the predicted noise level (LAeq,T) against the measured daytime LAeq,T level from the baseline survey (the ambient noise level measured over 10 days) has been used to provide context for the assessment of operational noise. This difference is considered to indicate the likelihood of a significant change in noise level at the noise sensitive receptors between the existing operations at FIPASS and those associated with the proposed scheme. As detailed in Table 12.27 and 12.29, the predicted operational phase noise level is far below the existing ambient noise level, with the FIPASS causeway noted to be a major contributor to the ambient noise level. The construction stage assessment, undertaken in accordance with British Standard 5228-1:2009+A1:2014, Code of Practice for Noise and Vibration Control on Construction and Open Sites (BS 5228-1), has considered SFM as a ‘residential receptor’ but concludes negligible impact to the Seafarer’s Mission during the daytime as the mission is sited in an existing industrial zone (ambient levels are thus high), and minor adverse impact is assessed at night time. A range of mitigation measures are to be adopted (see Section A12.3.1.1) to manage the construction noise disturbance, resulting in negligible residual impacts for a premises in an industrial zone FIG wishes to support the charitable work that the Mission is doing, especially for recuperating seafarers and we will therefore work to an enhanced noise standard than stated in the EIS. We will erect an acoustic barrier between our dismantlement works and the Mission, to ensure that important services are maintained and we limit our disruption to the facility during the works. The removal of the causeway, which is immediately adjacent to the Seafarer’s Mission, is likely to be one of the main reasons for the predicted decrease in ambient noise level during operation of the proposed scheme. In summary, negligible noise impacts are predicted during operation. Although users will have a longer journey to get to the Mission during operation compared to existing, the experience for users will be improved once there, compared to existing conditions. |
| 11 Does Surficial silt pipe route need to be along foreshore and cross TDF access | The pipe route is indicative at this stage and we will be discussing in detail with each land owner at discharge of condition stage the routing for the apparatus, controls required and access needs. It will be laid on the ground and foreshore where possible to avoid impact but will be buried to cross key accesses ( ie at the TDF approach) The route is proposed to avoid impact in the water with navigation routes onto FIPASS to ensure that shipping and cargo routes are not disrupted |
| FUEL TRANSFER | . |
| 12 Why are the Fuel pipelines below ground and not above? | These services are vulnerable to impact and resultant spillage (as proven on FIPASS in 2019) and it is important in sensitive marine ecology environments to ensure that risk of accidental spillage follows worldwide best practice and is risk is designed out where possible. It is a known fact that exposed pipelines are more vulnerable to damage, whether malicious or accidental, and burying them is considered general best practice. FIG is working closely with SSL and their designer Trident. SSL is undertaking the detailed design of the new fuel system themselves. These are a new specification to the system in place on FIPASS and are designed for simultaneous import and export and are at a different pressure to the existing services on FIPASS. The design is produced, checked and verified by SSL and provides a balanced approach of environment controls to design out spillage risks, improves functionality and future proofing for proposed power station as well. It also provides a robust and low maintenance asset for SSL in future. It is fully assessed at HAZOP and assessed for leakage risk and system failure. It also provides flexibility for SSL in the future, as fuel needs change within the Falkland Islands over the life of the project. The total quantity of fuel supplied to an individual vessels will be maintained in the new design, however it will be quicker and safer. The new port will be able to facilitate bunkering a vessel at the same time as the fuel tanker is alongside and transferring fuel to the Stanley Services fuel tank farm. This is not possible on FIPASS and is a significant efficiency improvement for port users. |
| WASTE MANAGEMENT |  |
| 13 What waste is to be recycled in the operational stage and the construction stage and what is to go to landfill? | In the operational stage the port will have 2 skips used for incinerator waste and inert non-combustible materials. This is as per advice from PWD on what will be provided on Island in the next 10 years. There is space allowed in the waste transfer zone for further recycling if further segregated waste streams become viable. Glass recycling and metal tin recycling can also be provided for the port office waste. Waste oil and bilge from vessels will be recycled by SSL and transported by IBC to SSL facilities. Non-recyclable oil from vessels will be transported off Island by the SSLTanker. Biosecurity materials will be stored separately when seized; double bagged and incinerated. In the construction stage the scheme is being designed to minimise waste streams, with packaging limited and constructed on island where possible. Existing soft strip materials are to be sent to land fill as this is environmentally the best option to avoid transportation by vessel where waste streams can be provided on Island. Emissions are assessed and include for waste transfer in the vehicle movements. All hazardous materials (ie hazardous and sludge) will be transported overseas to processing locations that can deal with these contaminates as capacity on Island is not developed for these waste streams. The main materials for recycling are steel from the barges. Refer to 22 and 23 for methodology and transportation |
| OTHER DESIGN QUERIES RAISED IN THE PLANNING APPLICATION STAGE |  |
| 14 Worker, visitor and pedestrian safety, and port lighting | The current design is compliant with international standards and recognised best practice for ports. The Safe System of Work (SSoW) currently is for the port to limit activity / close when wind speeds are not safe for staff and visitors, and this includes the cause way. This working practice will continue to ensure H & S risks are managed, especially for falling and for flying debris. Handrails will be provided along the causeway road in detailed design if the risk is assessed to need mitigating after the wind modelling review is complete. It is also important to consider safe access and unincumbered machine access for revetment maintenance, and balance these conflicting H & S risks. Life rings will be included on mobile stations at intervals of no greater than 100m. Ladders are spaced at maximum 30m centres with hand hold chains halfway in between. The new port has flood lighting proposed that brings the facility up to acceptable lighting for Health and Safety. It is designed to light the working surface, not the sky. The current lighting conditions on FIPASS are not bright enough for safe transhipment on the quay itself. Ship gear lights are regularly used for quayside task lighting due to inadequate lighting on the deck of FIPASS itself. Comments received during the Planning Application that suggest that the project proposes that ships are not lit for safety are misunderstanding the intent of the design for the quay lighting – see image provided. Each vessel has its own risk assessment on what it needs on board for safety purposes. The design clearly would not and could not dictate or predict Safe systems of work onboard individual vessels. The aim of the lighting design is to provide a dark sky ( ie not light the sky unnecessarily) and not to impact seabirds and marine life. The intent for the deck is below. Questions have also been raised on why e-coli risks need to be managed when people will need to work in these conditions for over a year. Certain concerns raised during Planning Application have suggested that material could be left in situ. To suggest that 50,000T of historic sewage is manageable and that protection for port users and workers is not required would represent a lack of regard to the serious health risks. Engineering assessment of risks relating to leaving the silt in situ was carried out during design development. The silt would present a long term risk to the effectiveness of the port surfacing as well if the surficial materials were not removed. It would generate differential settlement that would be very costly in the long term for FIG to maintain on an operational port. In addition, BAM would not be able to verify the construction of the quay surface as sound without removal of this material, and thus important warranties could not be provided to FIG. |
| 15 Mooring bollards – concern that the mooring design does not reflect needs of the Falkland’s quayside operations | This is a technical design issue as opposed to a planning issue. The mooring design is still ongoing and the type and design of bollards and fenders are to be agreed by the Harbour Master in conjunction with a specialist group of stakeholders, to ensure that local conditions and expertise is harnessed. The mooring design is part of the application being determined – information is indicative at this stage. The currently proposed bollard arrangements are fully compliant with all international standards and guidance. However, as a result of local conditions and circumstances, an updated bollard arrangement has been provided as part of the technical design as requested at a meeting in February 2022. The updated bollard layout being developed includes 100T double bitt bollards at approximately 18m centres, with 50T double bitt bollards in between. 150T storm bollards are also to be provided on the port, and 50T double bitt bollards are to be supplied at approximately 6m centres on the eastern and southern berths. |
| 16 Provision of LNG Fuel allowed for ? | There is no future provision for LNG fuel at the new port. This fuel is not currently available in the Falkland Islands and would need significant amendment of equipment, refrigeration and new tanks with the SSL fuel farm to accommodate this significant change. This was not identified as a need in the SHE stage and SSL have confirmed that this is the first query received for this fuel. |
| 17 Why are Improvements to Visitor Jetty not planned? | Any improvements to the main visitor jetty in Stanley are outside the scope of the New Port project. |
| EIA AND PLANNING PROCESS QUERIES |  |
| 18 Concern that the EIS submitted has not complied with all of FIG processes within the EIS for Marine Mammals? | The project complies with the policies and processes in place at the time of agreeing the scope of the EIA. Documents identified in a letter of concern relating to the Planning Application that relate to Marine mammals are newer documents produced after source documentation freeze for the purposes of the EIA. BAM have however confirmed that they will work to the new code of practice for Cetacean code of conduct for the Falkland Islands in the construction stage and this will be detailed in the EMP. It is envisaged that this will be controlled via a planning condition. The issues raised have no material effect to the EIS and planning application content before the committee. BAM has consideration experience in piling in proximity to Marine Mammals and has similar processes in place on projects in both Antarctica and South Georgia to protect marine mammals, agreed with the South Georgia Government and British Antarctic Survey (BAS). Measures proposed for the NPF are appropriate and in keeping with the detailed assessment of piling noise. Mitigation measures proposed are focused on monitoring and observing and soft start processes. BAM discussed and agreed as appropriate the MMO principles with the FIG Environmental Team, Natural Resources staff and an independent Ecology Advisor from Ramboll ahead of finalising the EIS for planning submission. |
| 19 Is the Quality and accuracy of the EIS acceptable? | The EIS is extensive and of a high standard. The EIS was informed by the publicly available information that was available at the time of writing. A detailed response to the specific comments made by Falkland Conservation was issued on 23/02/2022, which concluded that the new information available since production of the EIS would not materially affect the conclusions presented. In addition, the Marine Mammal Observation Plan provides the mechanism to manage the potential underwater noise impacts to marine mammals and therefore provides assurance that there are controls in place to manage potential impacts to marine mammals. A statement of competency was also issued by BAM to FIG on 23/02/2022. As detailed in the statement, Royal HaskoningDHV is the leading environmental consultant in the UK concerned with achieving consent for port and terminal developments. The planning committee and wider Falkland Islands public can therefore have confidence that the EIA has been undertaken by specialists who have extensive expertise and experience in this field. It should also be noted that some objectors appear to misunderstand how a EIS is calculated for environmental impact and what ‘negligible’ significance is. It is not determined by effect on individual businesses or isolated effects – rather it considers the whole subject ( i.e. Air quality for all areas affected) and is assessed from the base line of FIPASS as a starting point for the assessment, not on virgin conditions, as this is an assessment for a replacement to an existing facility. |
| 20 Why are habitats not enhanced by the new port facility? | There are no proposals for habitat reinstatement mitigation as a result of the scheme as the baseline is FIPASS and the main focus has been removal of significant hazards associated with the removal of the structure and its contaminates. The proposed use of the new rock armour will provide new habitat for colonisation by birds on the new rocky shelves and benthic colonisation in intertidal and subtidal zones. The pontoon to be constructed as part of the proposed scheme is to be located in the lee of the quay and causeway, and therefore the new pontoon is likely to offer a sheltered haul out location for marine mammals. The scheme has therefore incorporated measures by design to enhance habitat within the harbour. FIG have reviewed the request by Falkland Conservation in the planning determination process about habitat improvement on land around the new coastal path beyond the reprovision of the grass provided against the existing path. In response, FIG offers to improve and enhance the environment further through more extensive native planting and this will be actioned via a landscaping condition. |
| 21 Was timescales for public consultation and review of information, including public consultation adequate? | Full and comprehensive public engagement was undertaken initially in Stage 1A to inform development of the scheme with data on requirements of port users and future growth aspirations. Over 40 meetings were held initially with local businesses and port users and over 4000 responses to questions in questionnaires were taken into consideration to inform the design. FIG Planning Department’s Pre-Application advice took into consideration all the stakeholder engagement at the start of the project. No special changes to the process for this project were identified as necessary, in consideration too of the fact that the planning application and EIS is for a replacement port, not a new type of facility. In addition, FIG and BAM have engaged with many key stakeholders and users of the port through the design development process, as appropriate, to help inform ongoing design development. Further meetings were held during Stage 1A with various key stakeholders, and approximately 30 meetings more have been held with various stakeholders to develop the detailed design in Stage 1B. This process does not finish with the Planning Application, and will continue until end of detailed design. |
| DISMANTLEMENT |  |
| 22 What are the overall timescales for dismantling/cutting up FIPASS and what is the work sequence? | The soft strip and dismantlement of the top structures on FIPASS are to be undertaken whilst afloat. Removal of some of the hazardous materials will also occur from FIPASS, including fuel pipe purging and venting, pollution control cleaning where possible of the confined spaces to reduce explosive gas risk. Only the barge structures and the causeway themselves are to be dismantled on the land as indicated in the application and also the removal of residual sludge from the tanks. The overall process is long as the dismantlement is phased to ensure berthing is maintained on FIPASS until the new quay is partially completed and operational, and it includes for removal of hazardous materials. Each barge and linkspan is dismantled progressively with months of downtime between some of the removals. Some of the cutting activities are also undertaken in Area 3 (please refer to the main Planning Application drawings showing temporary working areas), which is the main recycling stock pile where materials are deconstructed to maximum sizes of 1.5m by 0.6 by 0.6 to allow for effective removal from Island. Each barge is programmed to take typically 48 days to complete once moved to the beach and the winching is started. Dismantlement will comply with UK standards for noise, damping down and cleaning off of benthic materials. The existing defunct RO-RO barge and causeway are the last to be dismantled and will take 100 days. |
| 23 Where is material to be stored whilst dismantlement occurs ? | It is predicted that the dismantling would generate approximately 15,000 tonnes of steel for recycling. The materials for recycling in the EIS include the warehouse superstructure, the dolphins not being reused, the barges themselves, their linkspans and the causeway structures As stated in Section A4.2.6.5 of the EIS, to avoid transportation to Mare Harbour, it is proposed that the steel which makes up the barges will be stored in a holding area which is area 3 in the planning pack [Area 1 in the EIS] and potentially other storage areas shown on Figure 4.1 in the EIS until the proposed new quay is operational. An area of approximately 1,000m2 will be required to store the scrap steel, once the material is cut up, which should easily be accommodated by the proposed temporary works compounds. Within the dismantling proposal, the steel for recycling will be transported to the new quay once operational and loaded onto a vessel for transport overseas where it is intended that the material will be recycled. |
| ENVIRONMENTAL RISKS OF DUST, SMELLS & NOISE |  |
| 24 How are airborne pollution from the dismantling works, including settlement on adjacent fields to be managed ? | The assessment of the dust and air impact to adjoining land is explained in Section A13 of the EIS. The assessment of impacts has been carried out in accordance with the UK Industry of Air Quality Management (IAQM) ‘Guidance on the assessment of dust from demolition and construction’. The scope of the air quality assessment was agreed with FIG’s Planning Department via the formal EIA scoping process. Measures are to be focused on best practice employed in reducing dust generation, mud on roads etc. It is recognised that the Stanley Growers land is adjacent to the proposed construction area. Although Stanley Growers land is in very close proximity to the construction area, the prevailing winds are westerly and debris will be damped down within the dismantlement area. Note that the method of dismantlement is by shears which minimalizes air borne particles in cutting compared to other methods This contributes to a low likelihood of dust deposition on Stanley Growers land and therefore an assessment of deposition on Stanley Growers land which is used for agriculture has not been undertaken. A range of mitigation measures have however been identified to manage / control the risk of air quality reductions, which are set out in Section A13.3.2.1. These will be detailed in a CEMP to prevent or minimise the release of dust and / or dust being deposited on nearby receptors. |
| 25 Will there be a smell from geo-tubes and why is this removal favourable? | Concerns raised regarding odour from the geotubes are noted. However, as explained in Section 4.2.6.8 of the EIS, the geo-tubes are very effective at capturing both suspended solids and the compounds which generate odour and therefore very minimal odour generation is expected from the sealed bags. These geo-tube solutions are used in countries with the highest environmental standards around the world, in habituated areas, which should provide assurance there are no significant issues around smell. The very limited gases not contained in the bag will blow away from the residential areas of Stanley, minimising any potential impacts on the local community. However as noted above, no significant odour impact is anticipated. This method of surficial silt removal is used worldwide and in areas where high UV levels are present. The solution is environmentally balanced to manage risk to marine environmental and water quality as well as shore and hinterland environment. Trying to keep the silt in the water has been reviewed and ruled out due to effect on navigation of banks of material migrating into berthing approach areas in the short and long term, ecology impact of significant silt flumes in the water column and effect to water quality in the harbour. Please also note that kelp removal is a separate activity to the surficial site removal in the sequence and is not accounted for in the bag volume except for smaller fragments that may enter the pump and can be dealt with by the pump blades. These fragments will bio-degrade in the bags along with all remaining invasives within the silt. |
| 26 Dismantling – Will it be noisy and are the figures right in the EIS? | The scope of the noise assessment (including the scope of the baseline noise survey) was agreed with FIG Planning and Building Services during the formal EIA scoping process. The monitoring locations were selected based on their proximity to existing and future receptors locations, and are considered appropriate to characterise the baseline noise environment. Review of findings from other baseline noise surveys from previous studies ( TDF) has also been used to understand the baseline environment and also to benchmark to non-COVID time period data. The noise assessment has been undertaken in accordance with the following British Standards and guidance: - Noise from temporary fixed and mobile plant (construction phase) - BS 5228-1:2009+A1:2014, Code of Practice for Noise and Vibration Control on Construction and Open Sites (BS 5228-1) (refer to Section A12.1.2.1 of the EIS) - Road traffic noise - guidance provided in Highways England Design Manual for Roads and Bridges (DMRB), (LA111 Revision 2, May 2020) (refer to Section A12.1.2.2 of the EIS). - Industrial and commercial noise (operational phase) - (BS 4142:2014+A1:2019 Methods for rating and assessing industrial and commercial sound (BS 4142) (refer to Section A12.1.3.1 of the EIS) The assessment, which has been undertaken in accordance with the above British Standards and guidance documents, concludes minor adverse impacts at worst during the construction phase and negligible impacts during operation. Although not considered necessary based on the assessment in the EIS, FIG has agreed to install a noise barrier around the Seafarer’s Mission during the construction phase of the proposed scheme refer to Question 10. As well as reducing the noise to the Seafarer’s Mission, this would also assist in minimising the visual disruption to the Mission. See Question 10 for more details. |
| 27 Dismantling - concerns with regard to air quality assessment and conclusions reached | Refer to Answer 24 above as well . The assessment of impact to air quality has been undertaken in accordance with the following: - Construction phase dust and plant emissions - UK Institute of Air Quality Management (IAQM) ‘Guidance on the Assessment of Dust from Demolition and Construction’ (IAQM, 2016) and UK’s Defra technical guidance (TG16) (Defra, 2018). - Construction and operational phase road traffic emissions - IAQM and Environmental Protection UK (EPUK) guidance (IAQM and EPUK, 2017). The assessment presented in Section A13.3.1 of the EIS concludes that emissions from Non-Road Mobile Machinery (NRMM) would be not significant to human receptors, taking into account the scheme being in an industrial zone. As detailed in Table 13.3, FIPASS is constructed of materials with a low potential for dust release (i.e. steel), and therefore the dismantling activity is unlikely to generate significant volumes of dust. The calculated dust magnitude is therefore small. In accordance with the guidance, the construction phase impacts on human health were described as ‘negligible risk’ for demolition and ‘low risk’ for earthworks, construction and track-out. A range of mitigation measures have however been identified to manage / control the risk of air quality reductions, which are set out in Section A13.3.2.1. These will be detailed in a CEMP to prevent or minimise the release of dust and / or dust being deposited on nearby receptors. |
| 28 How are Visual disturbance to neighbours of dismantlement site mitigated and what are the management controls ? | The concern from Seafarers Mission associated with visual disturbance during dismantling of the barges is noted. The area will be fenced to limit this. The assessment presented in Section A15.3.1 of the EIS considers the impact to receptors from the presence and movement of construction plant. The proposed plant is 2 excavators and a 250t crawler crane used for lifting sections only. The assessment presented in Section A15.3.1 is considered to be valid, in that the disturbance impact will be temporary only, with the impact being reversed on completion of the dismantling process. Comments received during the Planning Applicatin that reference ship dismantlement used in Asia are not valid. The processes that FIG and BAM will use are to UK standards and will be monitored and controlled robustly. The material is to be recycled and closely monitored for control and management of environmental hazards. As noted in the response to comment above, a noise barrier is to be installed around the Seafarer’s Mission (although as noted above this is not considered necessary based on the assessment in the EIS). As well as reducing the noise from the construction works, the barrier will also minimise any visual disturbance to the Seafarer’s within the Mission. Other boundaries will have chain link fencing for security of the boundary, including misadventure in non-working hours, and will also have debris netting attached where assessed as required. |
| 29 Will the odour be significant from the dismantlement of the FIPASS barges on the shore ? | There has been a suggested comparison with the SS Great Britain. Any comparison of a 1970 salvage operation of a vessel sunk for decades fully submerged in silt with significant benthic material on decks as well as on hulls is not a valid comparison to a modern dismantlement process of a working port structure with robust environmental controls in place. The ramp and cut off trench design and work sequence consider these points and the removal of the benthic material. Both dredge and surficial silt will be removed from the structure promptly one barge at a time and any residual bio-degrading materials will also be disposed of progressively in the sequence. |